高超声速流动；湍流；粗糙壁；可压缩性；图神经网络

Hypersonic flow; turbulence; rough wall; compressibility effect; graph neural network

基于图网络构建阻抗边界条件的高超声速粗糙壁湍流模拟研究

Numerical investigation of hypersonic turbulent flows over rough surfaces based on graph neural network featured impedance boundary condition

摘要：受加工或气动热烧蚀等因素影响，高超声速飞行器表面多呈粗糙状态，可压缩效应和粗糙壁综合作用产生的湍流场与光滑壁差异显著，其精确解析对高超声速飞行器的气动力和热防护设计至关重要。但受限于复杂的几何外形，相关的试验与模拟成本高昂，不易开展。对此，本项目提出一种结合图神经网络(GNN) 和时域阻抗边界条件(TDIBC)的简化计算方法，以实现高超声速粗糙壁湍流的简化模拟，并探究可压缩效应对高超声速粗糙壁湍流的影响。研究首先通过孔隙网络建模（PNM）构建粗糙壁几何拓扑与GNN的映射，通过训练得到节点阻抗模型，进而推导时域阻抗边界条件；发展粗糙壁凹陷区瞬时物理量的分布模型，结合TDIBC实现时精简化模拟；采用模态分析、Helmholtz分解等分析方法，探究可压缩效应对近壁面湍流的影响机理。拟开展的研究有助于提升高超声速飞行器表面流场的模拟效率，并为高超声速粗糙壁湍流提供理论支撑。

【背景】 + 【研究目标】+【研究内容】 + 【展望（创新点，上价值）】

Abstract: Due to the manufacturing process and ablation effect, surfaces of hypersonic vehicles often exhibit non-smooth textures. The turbulent flow field generated by a rough wall and compressibility effect significantly deviates from those over a smooth wall. The accurate resolving of such flow field is critical to the aerodynamic and thermo-protection design of hypersonic vehicles. However, it is difficult to carry out experimental and numerical studies due to the complicated surface geometry. This project proposes a simulation technique that combines graph neural network (GNN) and time-domain impedance boundary condition (TDIBC) that simplifies the calculation of hypersonic turbulent boundary layer flows over rough walls, and to explore the compressibility effect on the hypersonic near-wall turbulence. First, a topology connection between rough wall elements and GNN is constructed via the pore-network modeling (PNM), the impedance model is then trained, based on which TDIBC is established. Modal analysis and Helmholtz decompositions will be applied to the flow field to investigate the effect of compressibility effect on the near-wall turbulence. The proposed work aims at developing an efficient computational method for hypersonic boundary layer flow simulations, and also lay out the theoretical fundations of hypersonic turbulent flow over rough walls.