湍流;高超声速流动；；粗糙壁；可压缩性；图神经网络

turbulence simulation; Hypersonic flow;; rough wall; compressibility effect; graph neural network

基于图网络优化阻抗模型的高超声速粗糙壁湍流模拟与机理研究

Numerical investigation of hypersonic rough-wall turbulence based on GNN-optimized impedance model

摘要：高超声速飞行器表面受加工工艺、气动热烧蚀等因素影响多呈粗糙形态，可压缩效应和粗糙壁综合作用产生的湍流场与光滑壁差异显著，直接影响飞行器的气动力和热防护设计。传统试验与模拟方法因粗糙壁复杂几何特征成本高昂，制约了高超声速粗糙壁湍流的研究。对此，本项目提出一种结合图神经网络（GNN）和时域阻抗边界条件（TDIBC）的简化计算方法，突破传统方法对复杂粗糙壁几何的强依赖性，并探究可压缩效应对高超声速粗糙壁湍流的影响。研究首先通过孔隙网络建模（PNM）构建粗糙壁几何拓扑与GNN的映射，并构建节点阻抗模型和时域阻抗边界条件；发展粗糙壁凹陷区瞬时物理量的分布模型，结合TDIBC实现粗糙壁的时精简化模拟；采用模态分析、Helmholtz分解等方法，系统解析可压缩效应对近壁面湍流拟序结构演化和能量输运的影响机理。拟开展的研究有助于提升高超声速飞行器表面流场的模拟效率，并为高超声速粗糙壁湍流提供理论支撑。

【背景】 + 【研究目标】+【研究内容】 + 【展望（创新点，上价值）】

Abstract: Hypersonic vehicle surfaces often exhibit roughness due to factors such as manufacturing processes and aerodynamic thermal ablation. The turbulent flow field generated by the combined effects of compressibility and rough walls differs significantly from that of smooth walls, directly impacting the aerodynamic forces and thermal protection design of the vehicle. Traditional experimental and simulation methods require high costs due to the complex geometric of rough walls, limiting the research on hypersonic rough-wall turbulence. The proposed work provides a simplified computational method that combines Graph Neural Networks (GNN) and Time-Domain Impedance Boundary Conditions (TDIBC), alleviating the strong dependence of traditional methods on resolving complex rough-wall geometries. The study will also explore the effects of compressibility on hypersonic rough-wall turbulence. The project will first construct a mapping between the rough-wall geometric topology and GNN by using Pore Network Modeling (PNM), generate a data-driven node impedance model and then establish the corresponding TDIBC. A distribution model for instantaneous physical quantities of the rough wall will be developed, which will then be combined with TDIBC to achieve time-accurate simplified simulations of hypersonic rough walls. Modal analysis and Helmholtz decomposition will be utilized to systematically analyze effects of compressibility on the evolution of near-wall turbulent coherent structures and energy transport mechanisms. The proposed work aims to improve the simulation efficiency of the flow simulations over realistic hypersonic vehicle surfaces and provide theoretical support for hypersonic rough-wall turbulence.